! ============================================

! NETWORK TOPOLOGY

! ============================================

! Router1 (Gateway) --- Router2 --- Router3

!    |                    |           |

! Switch1              Switch2     Switch3

!  |   |                |   |       |   |

! PC1 PC2             PC3 PC4     PC5 PC6

!

! Private Networks:

! LAN1: 192.168.1.0/24 (PC1-PC2)

! LAN2: 192.168.2.0/24 (PC3-PC4)

! LAN3: 192.168.3.0/24 (PC5-PC6)

!

! Public IP: 203.0.113.1 (on Router1)

! EIGRP AS: 100

! ============================================

! ============================================

! ROUTER 1 CONFIGURATION (Gateway/Edge Router)

! ============================================

Router1>enable

Router1#configure terminal

! Configure hostname

Router1(config)#hostname Router1

! Configure interfaces

Router1(config)#interface GigabitEthernet0/0

Router1(config-if)#description WAN Interface (Public)

Router1(config-if)#ip address 203.0.113.1 255.255.255.252

Router1(config-if)#ip nat outside

Router1(config-if)#no shutdown

Router1(config-if)#exit

Router1(config)#interface GigabitEthernet0/1

Router1(config-if)#description LAN Interface to Switch1

Router1(config-if)#ip address 192.168.1.1 255.255.255.0

Router1(config-if)#ip nat inside

Router1(config-if)#no shutdown

Router1(config-if)#exit

Router1(config)#interface GigabitEthernet0/2

Router1(config-if)#description Connection to Router2

Router1(config-if)#ip address 10.0.12.1 255.255.255.252

Router1(config-if)#ip nat inside

Router1(config-if)#no shutdown

Router1(config-if)#exit

! Configure EIGRP Dynamic Routing

Router1(config)#router eigrp 100

Router1(config-router)#network 192.168.1.0 0.0.0.255

Router1(config-router)#network 10.0.12.0 0.0.0.3

Router1(config-router)#no auto-summary

Router1(config-router)#passive-interface GigabitEthernet0/0

Router1(config-router)#eigrp router-id 1.1.1.1

Router1(config-router)#exit

! Configure NAT/PAT (Port Address Translation)

! Define inside source network

Router1(config)#access-list 1 permit 192.168.1.0 0.0.0.255

Router1(config)#access-list 1 permit 192.168.2.0 0.0.0.255

Router1(config)#access-list 1 permit 192.168.3.0 0.0.0.255

Router1(config)#access-list 1 permit 10.0.12.0 0.0.0.3

Router1(config)#access-list 1 permit 10.0.23.0 0.0.0.3

! PAT configuration (overload) - Multiple internal IPs to one public IP

Router1(config)#ip nat inside source list 1 interface GigabitEthernet0/0 overload

! Static NAT example (optional) - Map PC1 to specific public IP

Router1(config)#ip nat inside source static 192.168.1.10 203.0.113.2

! Configure default route to ISP

Router1(config)#ip route 0.0.0.0 0.0.0.0 203.0.113.2

! Redistribute default route into EIGRP

Router1(config)#router eigrp 100

Router1(config-router)#redistribute static metric 10000 100 255 1 1500

Router1(config-router)#exit

Router1(config)#end

Router1#write memory

! ============================================

! ROUTER 2 CONFIGURATION (Internal Router)

! ============================================

Router2>enable

Router2#configure terminal

! Configure hostname

Router2(config)#hostname Router2

! Configure interfaces

Router2(config)#interface GigabitEthernet0/0

Router2(config-if)#description Connection to Router1

Router2(config-if)#ip address 10.0.12.2 255.255.255.252

Router2(config-if)#no shutdown

Router2(config-if)#exit

Router2(config)#interface GigabitEthernet0/1

Router2(config-if)#description LAN Interface to Switch2

Router2(config-if)#ip address 192.168.2.1 255.255.255.0

Router2(config-if)#no shutdown

Router2(config-if)#exit

Router2(config)#interface GigabitEthernet0/2

Router2(config-if)#description Connection to Router3

Router2(config-if)#ip address 10.0.23.1 255.255.255.252

Router2(config-if)#no shutdown

Router2(config-if)#exit

! Configure EIGRP Dynamic Routing

Router2(config)#router eigrp 100

Router2(config-router)#network 10.0.12.0 0.0.0.3

Router2(config-router)#network 192.168.2.0 0.0.0.255

Router2(config-router)#network 10.0.23.0 0.0.0.3

Router2(config-router)#no auto-summary

Router2(config-router)#eigrp router-id 2.2.2.2

Router2(config-router)#exit

Router2(config)#end

Router2#write memory

! ============================================

! ROUTER 3 CONFIGURATION (Internal Router)

! ============================================

Router3>enable

Router3#configure terminal

! Configure hostname

Router3(config)#hostname Router3

! Configure interfaces

Router3(config)#interface GigabitEthernet0/0

Router3(config-if)#description Connection to Router2

Router3(config-if)#ip address 10.0.23.2 255.255.255.252

Router3(config-if)#no shutdown

Router3(config-if)#exit

Router3(config)#interface GigabitEthernet0/1

Router3(config-if)#description LAN Interface to Switch3

Router3(config-if)#ip address 192.168.3.1 255.255.255.0

Router3(config-if)#no shutdown

Router3(config-if)#exit

! Configure EIGRP Dynamic Routing

Router3(config)#router eigrp 100

Router3(config-router)#network 10.0.23.0 0.0.0.3

Router3(config-router)#network 192.168.3.0 0.0.0.255

Router3(config-router)#no auto-summary

Router3(config-router)#eigrp router-id 3.3.3.3

Router3(config-router)#exit

Router3(config)#end

Router3#write memory

! ============================================

! SWITCH CONFIGURATIONS (Basic Layer 2)

! ============================================

! SWITCH 1

Switch1>enable

Switch1#configure terminal

Switch1(config)#hostname Switch1

Switch1(config)#interface range FastEthernet0/1-2

Switch1(config-if-range)#switchport mode access

Switch1(config-if-range)#switchport access vlan 1

Switch1(config-if-range)#no shutdown

Switch1(config-if-range)#exit

Switch1(config)#end

Switch1#write memory

! SWITCH 2

Switch2>enable

Switch2#configure terminal

Switch2(config)#hostname Switch2

Switch2(config)#interface range FastEthernet0/1-2

Switch2(config-if-range)#switchport mode access

Switch2(config-if-range)#switchport access vlan 1

Switch2(config-if-range)#no shutdown

Switch2(config-if-range)#exit

Switch2(config)#end

Switch2#write memory

! SWITCH 3

Switch3>enable

Switch3#configure terminal

Switch3(config)#hostname Switch3

Switch3(config)#interface range FastEthernet0/1-2

Switch3(config-if-range)#switchport mode access

Switch3(config-if-range)#switchport access vlan 1

Switch3(config-if-range)#no shutdown

Switch3(config-if-range)#exit

Switch3(config)#end

Switch3#write memory

! ============================================

! PC CONFIGURATIONS

! ============================================

! PC1 Configuration

IP Address: 192.168.1.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

DNS: 8.8.8.8

! PC2 Configuration

IP Address: 192.168.1.11

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

DNS: 8.8.8.8

! PC3 Configuration

IP Address: 192.168.2.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.2.1

DNS: 8.8.8.8

! PC4 Configuration

IP Address: 192.168.2.11

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.2.1

DNS: 8.8.8.8

! PC5 Configuration

IP Address: 192.168.3.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.3.1

DNS: 8.8.8.8

! PC6 Configuration

IP Address: 192.168.3.11

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.3.1

DNS: 8.8.8.8

! ============================================

! VERIFICATION COMMANDS

! ============================================

! On Router1, Router2, Router3 - Verify EIGRP

Router1#show ip eigrp neighbors

Router1#show ip eigrp topology

Router1#show ip route eigrp

Router1#show ip protocols

! On Router1 - Verify NAT/PAT

Router1#show ip nat translations

Router1#show ip nat statistics

Router1#show access-lists

! Verify full routing table

Router1#show ip route

! Test EIGRP adjacencies

Router2#show ip eigrp neighbors

Router3#show ip eigrp neighbors

! Debug EIGRP (use carefully)

Router1#debug eigrp packets

Router1#undebug all

! Test connectivity from PCs

PC1> ping 192.168.2.10

PC1> ping 192.168.3.10

PC1> ping 8.8.8.8

PC1> tracert 8.8.8.8

! ============================================

! EIGRP EXPLANATION

! ============================================

!

! EIGRP (Enhanced Interior Gateway Routing Protocol):

! - Cisco proprietary protocol (now open standard)

! - Advanced Distance Vector / Hybrid routing protocol

! - Uses DUAL algorithm for loop-free routing

! - Supports VLSM and CIDR

! - Fast convergence

! - Reduced bandwidth usage with incremental updates

! - Automatic route summarization (disabled with "no auto-summary")

!

! Key EIGRP Components:

! - AS Number: 100 (must match on all routers)

! - Router ID: Uniquely identifies each router

! - Metric: Based on bandwidth and delay

! - Passive Interface: Prevents EIGRP on WAN interface

!

! EIGRP Neighbor Requirements:

! - Same AS number

! - Same subnet

! - Authentication (if configured)

! - K-values match

!

! ============================================

! NAT/PAT EXPLANATION

! ============================================

!

! NAT (Network Address Translation):

! - Translates private IP addresses to public IP addresses

! - Static NAT: One-to-one mapping (e.g., PC1 always uses 203.0.113.2)

! - Dynamic NAT: Many-to-many mapping from a pool

!

! PAT (Port Address Translation / NAT Overload):

! - Multiple internal devices share ONE public IP address

! - Uses port numbers to distinguish connections

! - Most common in home/office networks

! - Configured with "overload" keyword

!

! In this configuration:

! - EIGRP dynamically advertises all internal routes

! - Default route is redistributed into EIGRP

! - PAT allows all 6 PCs to share the public IP 203.0.113.1

! - Static NAT gives PC1 a dedicated public IP 203.0.113.2

! - Router1 performs the NAT/PAT translation

! - All routers learn routes automatically via EIGRP

! - No static routes needed for internal networks

!

! ============================================

! EXPECTED EIGRP ROUTING TABLE

! ============================================

!

! Router1 should see:

! D    192.168.2.0/24 [90/...] via 10.0.12.2

! D    192.168.3.0/24 [90/...] via 10.0.12.2

! D    10.0.23.0/30 [90/...] via 10.0.12.2

!

! Router2 should see:

! D    192.168.1.0/24 [90/...] via 10.0.12.1

! D    192.168.3.0/24 [90/...] via 10.0.23.2

! D*EX 0.0.0.0/0 [170/...] via 10.0.12.1

!

! Router3 should see:

! D    192.168.1.0/24 [90/...] via 10.0.23.1

! D    192.168.2.0/24 [90/...] via 10.0.23.1

! D    10.0.12.0/30 [90/...] via 10.0.23.1

! D*EX 0.0.0.0/0 [170/...] via 10.0.23.1

!